SIMATS ENGINEERING

Department of Computer Science & Engineering

ASSESSMENT TOOL 1- EXPERIMENTS

| Course Code: | CSA1209 & CSA1215 | Course Title: | Computer Architecture |
| --- | --- | --- | --- |
| CO Assessed: | CO3 | Assessment: | ASSESSMENT TOOL 1- EXPERIMENTS |
| Weightage: | 40% | Total Marks: | 25 |
| CO3 Statement: | Design processor organization by constructing pipelined datapath structures, identifying hazard types (data, control, structural), and comparing superscalar techniques such as out-of-order execution, speculative execution, and simultaneous multithreading (SMT). (BL6) |  |  |
| Student Name: |  | Reg.No.: |  |
| Department: |  | Date: |  |

ANNEXURE A

- Design and simulate a system that performs register transfer operations along with basic arithmetic and logic operations such as addition, subtraction, AND, and OR. Use multiple registers to demonstrate how data is transferred between them during execution, and analyze the sequence of operations and the role of the ALU in processing data.

- Create a model of a computer system using single-bus and multi-bus organization, and perform data transfer operations between registers, memory, and I/O. Compare the time taken and efficiency of both approaches, and analyze how multiple bus structures improve parallel data transfer and reduce bottlenecks.

- Simulate a 5-stage instruction pipeline (Fetch, Decode, Execute, Memory, Write-back) and execute a set of instructions. Measure the performance in terms of speedup and throughput, and compare it with a non-pipelined system to evaluate the effectiveness of pipelining.

- Develop a pipeline execution scenario where data hazards, control hazards, and structural hazards occur. Observe their impact on instruction execution, and implement techniques such as stalling, data forwarding, and branch prediction to resolve these hazards and improve performance.

- Analyze a processor supporting superscalar execution, including out-of-order and speculative execution, and extend the study to include hyper-threading and simultaneous multithreading (SMT). Evaluate how multiple instructions are executed in parallel and compare the performance improvements in terms of instruction throughput and resource utilization.

Code of Conduct:

I __B. Samuel Jadon (192571042)_ (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

MARKS ALLOCATION

|  | Understanding of Concepts (20%) | Experimental Design (20%) | Use of Tools (25%) | Analysis of Results (20%) | Documentation (15%) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Understanding of Concepts | 25 | Demonstrates thorough understanding and effectively integrates multiple concepts/technologies | Good understanding with proper integration of most concepts | Basic understanding; limited or partial integration | Poor understanding; unable to integrate concepts |
| Experimental Design | 30 | Well-planned, systematic implementation with optimal solution | Correct implementation with minor issues | Basic implementation with errors or inefficiencies | Incorrect or incomplete implementation |
| Use of Tools | 20 | Effective and advanced use of tools/technologies with proper configuration | Appropriate use of tools with correct execution | Limited or inconsistent use of tools | Incorrect or minimal use of tools |
| Analysis of Results | 15 | Insightful analysis with accurate interpretation and validation of results | Good analysis with reasonable interpretation | Basic analysis with limited insights | Poor or incorrect analysis of results |
| Documentation | 10 | Clear, well-structured documentation with diagrams, code, and results | Organized documentation with necessary details | Basic documentation with missing details | Poor or incomplete documentation |

1.

A register transfer system can be designed using multiple registers such as R1, R2, R3, and R4 along with an Arithmetic Logic Unit (ALU). The registers temporarily store data, while the ALU performs arithmetic and logical operations.

Example operations:

- R1 ← R2: Transfers the contents of R2 into R1.

- R3 ← R1 + R2: The ALU adds R1 and R2 and stores the result in R3.

- R4 ← R1 − R2: The ALU subtracts R2 from R1 and stores the result in R4.

- R3 ← R1 AND R2: Performs a bitwise AND operation.

- R4 ← R1 OR R2: Performs a bitwise OR operation.

Execution sequence:

- Load data into R1 and R2.

- Transfer data between registers using control signals.

- Send selected register values to the ALU.

- The ALU performs the required arithmetic or logical operation.

- Store the ALU result in the destination register.

For example, if R1 = 15 and R2 = 5, addition produces R3 = 20, while subtraction produces R4 = 10. The ALU acts as the main processing unit for arithmetic and logical operations, while the registers provide fast temporary storage.

Register transfer operations allow data to move between registers, and the ALU processes this data according to the control signals generated by the control unit.

2.

A computer system can use either a single-bus or multi-bus organization to transfer data between registers, memory, the ALU, and I/O devices.

Single-Bus Organization:

In a single-bus system, all components share one common bus. Only one data transfer can normally take place at a time.

Example:

R1 → Bus → ALU → R2

If several operations are required, they must be performed sequentially. This makes the design simple and inexpensive but can create a bottleneck because all components compete for the same bus.

Multi-Bus Organization:

A multi-bus system uses two or more buses. Different data transfers can occur simultaneously.

Example:

R1 → Bus A → ALU ← Bus B ← R2

The ALU can receive two operands at the same time, allowing faster execution of arithmetic operations. Memory and I/O transfers can also be organized more efficiently.

| Feature | Single-Bus | Multi-Bus |
| --- | --- | --- |
| Number of buses | One | Two or more |
| Data transfers | Mostly sequential | More parallel |
| Speed | Lower | Higher |
| Hardware cost | Lower | Higher |
| Bottleneck | More likely | Reduced |
| Complexity | Simple | More complex |

Multi-bus organization provides better performance because multiple data transfers can occur in parallel. However, it requires additional hardware and control circuitry compared with a single-bus system.

3.

A five-stage instruction pipeline divides instruction execution into five stages:

- Fetch (IF) – The instruction is fetched from memory.

- Decode (ID) – The instruction is decoded and registers are identified.

- Execute (EX) – The ALU performs the required operation.

- Memory (MEM) – Memory is accessed if required.

- Write-back (WB) – The result is written into the destination register.

Consider four instructions: I1, I2, I3, and I4.

Pipeline execution:

| Cycle | I1 | I2 | I3 | I4 |
| --- | --- | --- | --- | --- |
| 1 | IF |  |  |  |
| 2 | ID | IF |  |  |
| 3 | EX | ID | IF |  |
| 4 | MEM | EX | ID | IF |
| 5 | WB | MEM | EX | ID |
| 6 |  | WB | MEM | EX |
| 7 |  |  | WB | MEM |
| 8 |  |  |  | WB |

Without pipelining, each instruction requires five stages to complete before the next instruction starts. Therefore, four instructions require approximately:

Non-pipelined time = 4 × 5 = 20 cycles

With pipelining:

Pipelined time = 5 + (4 − 1) = 8 cycles

The ideal speedup is:

Speedup = Non-pipelined time / Pipelined time

Speedup = 20 / 8 = 2.5

Throughput is also improved because after the pipeline is filled, approximately one instruction can complete every clock cycle.

Conclusion: Pipelining improves instruction throughput and reduces the average execution time per instruction, although hazards and pipeline stalls can reduce the actual speedup.

4.

Pipeline hazards are situations that prevent the next instruction from executing in the intended clock cycle. The three major types are data hazards, control hazards, and structural hazards.

1. Data Hazard

A data hazard occurs when an instruction depends on the result of a previous instruction.

Example:

I1: ADD R1, R2, R3

I2: SUB R4, R1, R5

I2 needs the value of R1 produced by I1. This can be resolved using data forwarding, where the result from the ALU is directly forwarded to the next instruction without waiting for write-back. If forwarding is not sufficient, a stall is inserted.

2. Control Hazard

A control hazard occurs because of branch or jump instructions.

Example:

BEQ R1, R2, TARGET

The processor may not know which instruction to fetch until the branch condition is evaluated. Branch prediction can predict whether the branch will be taken. Correct predictions avoid stalls, while incorrect predictions require the incorrectly fetched instructions to be discarded.

3. Structural Hazard

A structural hazard occurs when two instructions require the same hardware resource at the same time. For example, if instruction fetch and memory access require the same memory unit, a conflict occurs.

It can be resolved by:

- Adding additional hardware resources.

- Using separate instruction and data caches.

- Stalling one of the instructions.

Conclusion: Stalling prevents incorrect execution but reduces performance. Data forwarding reduces data-hazard stalls, while branch prediction reduces control-hazard delays. Additional hardware resources help eliminate structural hazards and improve pipeline efficiency.

5.

A superscalar processor can execute multiple instructions during a single clock cycle by using multiple execution units such as ALUs, floating-point units, and load/store units.

Superscalar Execution:
Multiple independent instructions are issued and executed simultaneously. For example, two instructions can execute in parallel if they do not depend on each other.

Out-of-Order Execution:
Instructions are executed when their required operands and execution units are available rather than strictly following program order. This keeps processor resources busy and improves instruction throughput.

Speculative Execution:
The processor predicts future instruction paths, especially branches, and executes instructions before it knows whether the prediction is correct. If the prediction is wrong, the speculative results are discarded.

Hyper-Threading / SMT:
Simultaneous Multithreading (SMT) allows a single physical processor core to maintain and execute instructions from multiple threads. When one thread is waiting for memory or another resource, instructions from another thread can use available execution units.

Performance comparison:

| Technique | Main Advantage |
| --- | --- |
| Superscalar | Executes multiple instructions per cycle |
| Out-of-order execution | Reduces idle execution-unit time |
| Speculative execution | Reduces branch-related delays |
| SMT/Hyper-Threading | Improves utilization of a processor core |
| Combined approach | Increases overall instruction throughput |

For example, a processor capable of issuing four independent instructions per cycle has a theoretical issue width of 4 instructions/cycle. In practice, dependencies, cache misses, branch mispredictions, and resource conflicts reduce the actual throughput.

Conclusion: Superscalar execution increases parallelism, out-of-order execution improves resource utilization, speculative execution reduces control-flow delays, and SMT allows multiple threads to share processor resources. Together, these techniques significantly improve instruction throughput and processor utilization compared with a basic single-issue processor.