SIMATS ENGINEERING

Department of Computer Science & Engineering

ASSESSMENT TOOL 1- INDUSTRY PROBLEM SOLVING TASK

| Course Code: | CSA12 | Course Title: | Computer Architecture for Multicore Systems |
| --- | --- | --- | --- |
| CO Assessed: | CO4 | Assessment: | ASSESSMENT TOOL1- INDUSTRY PROBLEM SOLVING TASK |
| Weightage: | 30% | Total Marks: | 25 |
| CO4 Statement: | Implement hierarchical memory systems by differentiating cache levels (L1, L2, L3), virtual memory with paging, and advanced memory technologies (DRAM, SRAM, NAND Flash, MRAM, RRAM) based on latency, bandwidth, and throughput characteristics. |  |  |
| Student Name: |  | Reg.No.: |  |
| Department: |  | Date: |  |

ANNEXURE A

INDUSTRY PROBLEM SOLVING TASK

- Smartphone Performance Optimization                                 
 A mobile manufacturer observes lag while switching between apps. Analyze the role of L1, L2, L3 cache and DRAM and propose a hierarchical memory redesign to reduce latency and improve throughput.

- Cloud Server Memory Bottleneck
 A cloud service provider experiences high latency in database queries. Compare cache hierarchy vs virtual memory paging and recommend improvements using appropriate memory technologies.

- Autonomous Vehicle Data Processing
 An autonomous vehicle must process sensor data in real time. Evaluate SRAM, DRAM, and MRAM and design a memory hierarchy that maximizes bandwidth and minimizes latency.

- AI Inference Edge Device
 An edge AI device needs fast model loading with low power consumption. Compare NAND Flash, DRAM, and RRAM and recommend an optimized hierarchical memory structure.

- High-Performance Gaming Console
 A gaming console suffers frame drops during large scene loading. Analyze L3 cache vs DRAM throughput and propose memory hierarchy enhancements.

Code of Conduct:

I __B. Samuel Jadon(192571042)___ (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

MARKS ALLOCATION

|  | Understanding of Industry Problem (20%) | Application of Engineering Concepts (25%) | Solution Design (25%) | Use of Tools (15%) | Analysis (10%) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Understanding of Industry Problem | 20% | Comprehensive understanding of real-world problem and constraints | Good understanding with minor gaps in requirements | Basic understanding; some requirements unclear | Poor understanding; misinterprets problem context |
| Application of Engineering Concepts | 25% | Applies advanced and relevant concepts effectively to solve the problem | Applies appropriate concepts with minor errors | Limited application of concepts | Incorrect or no application of concepts |
| Solution Design | 25% | Innovative, practical, and feasible solution aligned with industry standards | Logical and feasible solution with minor gaps | Basic solution with limited feasibility | Unclear or impractical solution |
| Use of Tools | 15% | Effective use of modern tools, technologies, and real data | Appropriate use of tools with correct implementation | Limited or inconsistent use of tools | Minimal or incorrect use of tools |
| Analysis | 10% | Thorough analysis with validated results and performance evaluation | Good analysis with reasonable validation | Basic analysis with limited validation | Poor or no analysis |

1.

A smartphone manufacturer observes lag when users switch between multiple applications. This can occur when the processor frequently accesses slower DRAM because the required application data is not available in the processor cache. Cache misses increase memory access latency and reduce overall system performance.

Role of L1, L2, L3 Cache and DRAM

L1 Cache:
L1 is the smallest and fastest cache and is located closest to the CPU cores. It stores frequently accessed instructions and data, providing very low access latency.

L2 Cache:
L2 is larger than L1 but slightly slower. It stores data that cannot fit in L1 and reduces the number of accesses to slower memory.

L3 Cache:
L3 is larger and shared among multiple CPU cores. It helps maintain frequently shared application data and reduces accesses to DRAM.

DRAM:
DRAM provides large main-memory capacity but has significantly higher latency than cache. Applications that are not actively stored in cache must be accessed from DRAM.

Proposed Memory Hierarchy

CPU Cores

↓

┌───────────────┐

│   L1 Cache    │

└───────┬───────┘

↓

┌───────────────┐

│   L2 Cache    │

└───────┬───────┘

↓

┌───────────────┐

│   L3 Cache    │

└───────┬───────┘

↓

┌───────────────┐

│     DRAM      │

└───────────────┘

The redesigned system should use sufficiently sized L2 and L3 caches to retain frequently used application data. A high-bandwidth mobile DRAM technology can be used as the main memory.

Expected Improvement

A larger and more efficient cache hierarchy increases the cache hit rate and reduces expensive DRAM accesses. This decreases application switching latency and improves CPU throughput. Cache-aware scheduling and efficient memory management can further improve performance.

An optimized L1-L2-L3 cache hierarchy combined with high-bandwidth DRAM provides faster access to frequently used application data. This reduces latency during application switching and provides a smoother smartphone experience.

2.

A cloud service provider experiences high latency during database queries. Database workloads frequently access large amounts of data, and performance can be affected by cache misses, insufficient RAM, and excessive virtual memory paging.

Cache Hierarchy vs Virtual Memory Paging

| Feature | Cache Hierarchy | Virtual Memory Paging |
| --- | --- | --- |
| Purpose | Reduces CPU memory access latency | Extends available memory |
| Speed | Very high | Much slower |
| Storage | SRAM | DRAM + secondary storage |
| Main benefit | Faster repeated data access | Allows larger programs/data |
| Problem | Cache misses | Page faults |
| Impact on database | Improves frequently accessed queries | Excessive paging increases latency |

Cache memory is useful for storing frequently accessed instructions and data close to the CPU. Virtual memory uses pages and allows applications to use more memory than the available physical RAM, but accessing storage during a page fault is much slower than accessing DRAM.

Proposed Memory Architecture

CPU

↓

L1 Cache

↓

L2 Cache

↓

Shared L3 Cache

↓

Large DRAM

↓

High-Speed SSD / NVMe Storage

The server should be equipped with sufficient high-speed DRAM so that frequently accessed database pages remain in physical memory. A large shared L3 cache can reduce CPU memory access latency.

Database indexes should also be optimized so that queries require fewer memory accesses. Memory-intensive databases can benefit from keeping frequently accessed indexes and hot data in RAM.

Recommended Improvements

- Increase physical DRAM capacity.

- Use high-bandwidth DDR memory.

- Increase shared L3 cache where supported.

- Reduce unnecessary virtual memory paging.

- Use fast NVMe SSDs for unavoidable page faults.

- Optimize database indexes and query execution.

- Keep frequently accessed database pages in memory.

Expected Improvement

The main goal should be to minimize page faults because accessing storage is much slower than accessing DRAM. Increasing DRAM capacity keeps more database data in memory, while the cache hierarchy reduces CPU access latency.

Cache hierarchy and virtual memory solve different problems. Cache improves processor access speed, while virtual memory provides memory capacity. For a database server, increasing DRAM capacity and optimizing the cache hierarchy should be prioritized, while high-speed NVMe storage should be used as a backup for unavoidable paging.

3.

An autonomous vehicle continuously receives data from cameras, LiDAR, radar, GPS, and other sensors. This data must be processed in real time. High memory latency can delay object detection, path planning, and decision-making.

Comparison of SRAM, DRAM and MRAM

| Memory | Speed | Capacity | Power | Main Application |
| --- | --- | --- | --- | --- |
| SRAM | Very high | Low | Relatively high per bit | CPU cache |
| DRAM | High | High | Moderate | Main memory |
| MRAM | High | Medium | Low standby power | Persistent/fast storage |

SRAM provides the lowest latency and is suitable for processor caches, but its high area and cost limit its capacity. DRAM provides large capacity and high bandwidth and is suitable for storing sensor and processing data. MRAM is non-volatile and can retain data without continuous power.

Proposed Memory Hierarchy

AI / CPU Processing Cores

↓

SRAM L1/L2

↓

Large SRAM / L3

↓

High-Bandwidth DRAM

↓

MRAM

↓

Persistent Storage

Frequently accessed sensor data and AI computation data should be stored in SRAM caches. Large sensor buffers, intermediate results, and AI model data should be stored in high-bandwidth DRAM.

MRAM can be used for persistent system information, configuration data, and selected frequently accessed data that benefits from non-volatility.

Performance Optimization

The system should prioritize:

- Very low latency for control-critical operations.

- High bandwidth for camera and LiDAR data.

- Efficient data movement between memory levels.

- Parallel memory access.

- Local caching of frequently accessed sensor data.

The AI processor can use SRAM to store frequently accessed neural-network parameters and intermediate calculations, while DRAM stores larger models and sensor datasets.

Expected Improvement

Using SRAM for latency-critical operations and high-bandwidth DRAM for large sensor datasets provides a balance between speed and capacity. MRAM adds non-volatile storage with low standby power.

A hybrid SRAM-DRAM-MRAM architecture is suitable for autonomous vehicles. SRAM minimizes latency, DRAM provides the required bandwidth and capacity, and MRAM provides energy-efficient non-volatile storage.

4.

An edge AI device needs to load machine-learning models quickly while operating under strict power constraints. Traditional storage such as NAND Flash provides high capacity and non-volatility but has higher access latency than DRAM.

Comparison of NAND Flash, DRAM and RRAM

| Feature | NAND Flash | DRAM | RRAM |
| --- | --- | --- | --- |
| Volatile | No | Yes | No |
| Speed | Moderate | Very high | High |
| Capacity | Very high | High | Medium |
| Power | Low standby | Higher | Low |
| Main use | Permanent storage | Working memory | Fast non-volatile memory |

NAND Flash is suitable for permanently storing AI models, operating systems, and application data. DRAM provides fast working memory for model execution. RRAM can provide fast non-volatile storage and can reduce the need to reload frequently used data from slower storage.

Proposed Memory Hierarchy

AI Accelerator / CPU

↓

On-chip SRAM

↓

DRAM

↓

RRAM

↓

NAND Flash

The AI model should be permanently stored in NAND Flash. Frequently used models or model components can be placed in RRAM. During inference, the active model and intermediate data should be loaded into DRAM and on-chip SRAM.

Power Optimization

The system can reduce power consumption by:

- Keeping inactive NAND Flash in low-power states.

- Loading only required model components.

- Using SRAM for frequently accessed data.

- Using RRAM for frequently reused non-volatile model data.

- Reducing unnecessary data movement between memory levels.

Expected Improvement

This hierarchy provides a balance between capacity, speed, and energy efficiency. NAND Flash provides large permanent storage, RRAM provides faster non-volatile access, and DRAM provides high-speed working memory.

A combination of NAND Flash, RRAM, DRAM, and on-chip SRAM provides an efficient memory hierarchy for edge AI devices. It enables fast model loading while reducing unnecessary power consumption.

5.

A gaming console may experience frame drops when loading large game scenes because large textures, 3D models, animations, and other assets must be transferred from main memory or storage to the processor and GPU. Insufficient memory bandwidth can prevent the GPU from receiving data quickly enough.

L3 Cache vs DRAM

| Feature | L3 Cache | DRAM |
| --- | --- | --- |
| Technology | SRAM | DRAM |
| Capacity | Smaller | Much larger |
| Latency | Very low | Higher |
| Bandwidth | High | Very high overall capacity bandwidth |
| Cost per bit | High | Lower |
| Main purpose | Frequently used data | Large datasets and game assets |

L3 cache provides fast access to frequently reused data. However, large game scenes contain much more data than can fit inside the cache. Therefore, high-bandwidth DRAM is essential for transferring large textures and other assets.

Proposed Memory Hierarchy

CPU / GPU

↓

L1 / L2 Cache

↓

Shared L3 Cache

↓

High-Bandwidth DRAM

↓

NVMe Storage

The console should use a large and efficient shared L3 cache for frequently reused game data. High-bandwidth DRAM should be used to provide continuous data to the CPU and GPU.

A high-speed NVMe-based storage system can rapidly load large game assets into DRAM. Direct memory access and optimized data streaming can reduce CPU overhead.

Performance Enhancements

- Increase DRAM bandwidth.

- Optimize shared L3 cache usage.

- Use high-speed NVMe storage.

- Prefetch frequently required game assets.

- Use DMA for efficient data transfers.

- Stream large assets before they are required.

- Optimize memory allocation between CPU and GPU.

Expected Improvement

A larger and more efficient cache reduces repeated DRAM accesses for frequently used data. High-bandwidth DRAM allows large game assets to be transferred quickly, while fast storage and prefetching reduce loading delays.

L3 cache and DRAM have different roles. L3 cache minimizes latency for frequently accessed data, while high-bandwidth DRAM provides the capacity and throughput needed for large game scenes. Combining an efficient cache hierarchy, high-bandwidth DRAM, and fast NVMe storage can reduce frame drops and improve gaming performance.